The instructions are given below to simplify the implementation of the project. However, one might have different opinions on implementation. Hence, the instructions should not be taken as “final” to complete the project.

Things to remember:

- You will implement the individual module, test/debug and integrate all modules. So, start early!

- Carefully read the link description and files’ details as you move on.

- Don’t delete any code, you need them (rinse and repeat). Moreover, if your system integration does not work as requirement posted, you would be given partial points depending on the percentage of the project you complete. Take this opportunity and learn without overly being concerned about the grade.

Module Description

- Picoblaze:

  - Verilog code from PSM file should control IO from the user.

  - To create the GUI for five commands, use Picoblaze and UART protocol.

  - On a high level, figure out the sequential pattern(s) that can occur in a state. For example, what a user can do after record a message (then h/she can play, pause, stop, delete individual or all, etc.). You can use dip switches or buttons to control these functions.

  - Recoding would be stopped when the last memory address has been reached. Similarly, the memory would be reclaimed if any message is deleted.

  - Try with one message first. If all available functions (play, delete, etc.) to that message work, move on multiple messages. (Again, we are not recoding yet. The GUI should work first before we move on to audio codec and memory).

  - Depending on user input, we can wait on for the next input or wait until the current operation is finished (e.g. recording).

- RAM

  - Perform message playback (readout digital audio stored) and supply to the audio codec.

  - Accept/store the digital output from the audio codec. The messages are analog.

  - After each successful storing of the message, increment the address.

  - The controlling states within RAM should be enough to act on user input that goes picoblaze first, then top module followed by RAM again.

  - An ideal RAM FSM should have read/write capability with corresponding set/reset address.

- Top module:

  - Glues all modules. Should be implemented as a controller.

  - Picoblaze takes user input and sends to the top module which communicates with RAM for necessary action(s).

------------------------------------------------------------------------------------------------------------------------------------------

Details on Requirement:

- Play a message:

  - On each track:

    - Play, pause, resume, skip (what you will do if you select last track!)

- Recode a message:

  - Check for memory availability

    - If free, record.

    - If full, delete based on request, then record.

    - Set the flag accordingly.

- Delete a message:

  - Check if any message is stored in memory. If present:

    - Delete, reclaim the space.

- Delete all messages:

  - If message(s) exist, delete all and reclaim all messages’ space.