Person 1 Audio Codec Answers

Kaushik Selvakumar - CDA 4203/4203L Final Project

Project: Audio Message Recorder on Digilent Anvyl FPGA

# Lab 6 Connection

Lab 6 is related to this final project mainly through picoBlaze, UART, and the PuTTY serial-terminal workflow. The Lab 6 shared-folder files include the program.psm assembly pattern, kcpsm6.v, UART modules, and loopback top-level design. Those files are most directly useful for Person 3, who is handling the serial menu and command interface.

For Person 1, the implementation is based on the final-project audio codec demo rather than the Lab 6 UART logic. The audio module exposes clean sample and control ports so the Lab 6-style picoBlaze UI can control record/playback through the final top module.

# Answers For Claudio

| Question | Answer |
| --- | --- |
| Are you keeping sockit_top.v as the audio wrapper? | Yes. I am keeping the same audio-wrapper architecture from the Canvas sockit_top.v, but I moved it into an integration-ready wrapper named audio_frontend.v. |
| What exact signal gives the microphone sample? | record_sample_16[15:0] gives the microphone sample. For Joaquin's current 8-bit memory controller, use record_sample_8[7:0], which is the upper byte of the 16-bit sample. |
| What exact signal receives the speaker/playback sample? | playback_sample_16[15:0] receives the full playback sample. If we keep the current 8-bit memory controller, drive playback_sample_8[7:0] and set playback_use_8bit = 1'b1. |
| Are we using sample_end[1] and sample_req[1]? | Yes. audio_frontend.v exposes these selected-channel signals as sample_end and sample_req. Store microphone data when sample_end pulses, and provide the next playback sample when sample_req pulses. |
| What files should Person 4 add to the final ISE project? | Add src/audio_frontend.v, src/audio_codec.v, src/i2c_av_config.v, src/i2c_controller.v, and src/clk_wiz_v3_6.vhd. Merge constraints/audio_frontend_pins.ucf into the final system UCF. Do not add sim/sim_clk_wiz_v3_6_stub.v to the hardware project; it is only for Icarus Verilog syntax checks. |

# Current Folder Structure

src/ contains the ISE hardware source files.

constraints/ contains UCF files for standalone testing and final integration.

docs/ contains this document and the Markdown answers.

sim/ contains the Icarus-only clock wizard stub.

canvas_reference/audio_codec_demo/ contains relevant original Canvas audio demo files for traceability.

# Dependency On Other Teammates

Person 1 can be developed and tested independently in microphone passthrough mode. Full record/playback behavior depends on the final top module connecting Person 1 audio ports to Person 2 memory and Person 3 picoBlaze commands.

Person 2 must confirm whether the final memory path stores 8-bit or 16-bit audio samples.

Person 3 must confirm command codes for record, play, pause, delete, delete all, and volume.

Person 4 must provide the final clock wiring and instantiate audio_frontend in the combined top module.

Volume control can be added in audio_frontend once the final volume command/level signal is defined.